WDSS and DataVest

A Great Combination for the Amusement Park

To carry the WDSS unit in a DataVest, one needs to hold it so the Start/Stop button is readily accessible, and so the indicator LED is visible. The technique described below is relatively easy and yields a good result.

Materials:

Foam – 1.5” x 10” x 4.5”

Cardboard – 4.5” x 10”

– 2” x 5”

Carpet Tape (double-sided)

Tools:

Sharp knife (wallpaper knife). Electric knife, Scissors, Straight edge

Preparation:

- Obtain foam that is the size you need or cut some to size.

- Mark the outline of where you will insert the WDSS unit. This size can be a bit smaller than the actual size of WDSS in order to hold it securely.

- Using the wallpaper knife and the electric knife, cut out the space for the WDSS. In addition, add a slit at the top into which the force probe mounting post can fit.

- Cut cardboard pieces to size. The cardboard does not need to be particularly stiff.

- Put two or more lines of carpet tape along the larger cardboard piece as shown in the photo on the next page.

Assembly:

- Remove the protective covering from the carpet tape and press the foam onto the backing, securing it.

- Put the smaller cardboard piece inside the opening in the foam, sticking it to the backing.

- Insert the WDSS unit.

- Slide the entire assembly into the pocket on a DataVest. You will find you can operate the Power switch through the material, and should be able to clearly see what is happening with the WDSS unit while you are on a ride.

Use:

For instructions on use of the WDSS in the context of the amusement park, go to this url and download the document called Data Collection at the Amusement Park.

http://www2.vernier.com/booklets/data_park.pdf

Appendix I will give you step-by-step instructions for using WDSS at the park.

May 2007