TRIANGULATION INSTRUMENT AND ACCELEROMETER

- Glue this
 page to 
 manila paper.

- Cut out along 
the dashed line.

- Fold the top section
over a pencil and roll it
 down to the dotted line to 
make a sighting tube.

- Tape the rolled paper tube then let 
the pencil fall out.

- Take about 10 inches of sting and tie one end 
to a weight (a rubber hose washer is ideal). Tie the 
other end through the hole at the top of the drawing.

- Let the string hang free. The angle it marks off is the angular height of an object seen through the tube.

- This device can be used for measuring heights, and as a horizontal accelerometer. The acceleration value is simply g tan , where g is the acceleration of gravity.