Enterprise Exploration

Introduction

A number of amusement park rides with various names are derived from the original “Enterprise”. The photo to the right shows a view of the ride in operation.

Passengers get into the cars a short distance above the ground, with the main portion of the ride mechanism horizontal to the ground. The ride begins rotating faster and faster and the cars move outward, almost reaching horizontal. Why?

When the ride reaches its maximum angular velocity, the whole ride tilts approaching a vertical circle. After reaching its highest angle, the ride returns to level and brakes are applied to end the experience.

While measurements taken in this lab are done by an accelerometer, the results are a measure of forces that would be felt by a rider in the same situation. Explain how acceleration and force are related.

In this lab, our initial interest is to examine the accelerations when the ride is rotating in a horizontal plane and then approaching a vertical plane.

Figure 1

objectives

- Examine the accelerations experienced by the riders on an amusement park ride known as “Enterprise” while it is rotating in a horizontal plane.

- Compare the forces experienced as Enterprise tilts up towards a vertical plane.

Materials

Chromebook, computer, or mobile device

Graphical Analysis 4 app

GDX-ACC, GDX-FOR, WDSS or other accelerometer compatible with GA 4

Bicycle Gyroscope

Procedure

- Attach the accelerometer to your bicycle gyroscope with the long axis of the body aligned along a radius. (A regular bicycle wheel could be used with some care in making sure the accelerometer doesn’t hit the forks or the brakes as the wheel rotates.) A good practice is to apply a counterweight at the same radius on the opposite side of the axle.

- Measure the radius of the accelerometer cell from the axle of the wheel. The accelerometer is just beneath the x-y-z axis diagram. Record in your data table.

- Launch Graphical Analysis. Connect and set up the accelerometer for your Chromebook, computer or mobile device:

- Click or tap on New Experiment. Choose Wireless Devices.

- Click or tap on the accelerometer that appears such as GDX-ACC.

- Click or tap on the circled “i” for sensor information.

- Click or tap on Sensors, then the “+” in front of Z-axis gyro to add this to X-axis acceleration.

- Go back to Device > Wireless Devices > Done. You should now be in the graph display screen.

- Click or tap on the X-axis acceleration icon at the bottom of the graph screen. With the accelerometer held in a horizontal position, click or press Zero to set a zero value with the accelerometer in a horizontal position. Repeat for the Z-axis gyro.

- Data collection is set to end after 60 seconds. This is likely to be way too long. Click or tap on Mode and set a time that works for you like perhaps 6 s.

- Finally, click or tap on the display icon to set the display to 1 Graph.

- Click or tap on Done. You are now set to carry out this exploration.

- Place the gyroscope wheel so it revolves with the accelerometer in a level plane. Give the wheel a gentle push so it rotates approximately once every 2 to 3 seconds.

- Collect Data: Click or tap Collect to start data collection. After you have accumulated several rotations of the wheel, click or tap Stop to conclude data collection. If the time you set runs out the collection will stop by itself.

- Study your graphs to determine if the run was useful. You should see clearly the acceleration vs. time graph as the accelerometer moved around in its circular path.

- Click or tap and drag across a portion of the graph involving 3-4 cycles of motion. Use the Graph Tools button to then choose View Statistics. What is the average value of the acceleration?

- Click or tap on the vertical axis label, and change to x-angular velocity. Uncheck the x-axis acceleration graph. What is the average value of the angular velocity w?

- The linear acceleration is given by Rw2, where R is the radius of the motion. Do this calculation and compare your answer to the value determined in 6a.

- Repeat as requested by your instructor with different angular speeds.

- Change the orientation of the bicycle gyroscope so the rotation is in a vertical plane. Collect data for 5-6 rotations, then conduct these analysis steps.

- Click or tap and drag across a portion of the graph involving 3-4 cycles of motion. Use the Graph Tools button to then choose View Statistics. What is the average value of the acceleration at the top of the motion? What is the average value at the bottom of the motion?

- Show algebraically that the difference in acceleration at the top and bottom should just be equal to twice the acceleration due to gravity, 2g.

- Test the theory using data you gathered in this step.

- Repeat with different angular speeds as requested.

Data Table

| Radius of accelerometer | m |
| --- | --- |

| Horizontal Trial | Average acceleration a (m/s2) | Angular velocity w (rad/s) | Calculated acceleration w2/R (m/s2) | Percentage difference |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

| Vertical Trial | Acceleration at top a (m/s2) | Acceleration at bottom a (m/s2) | Acceleration difference Da (m/s2) | Compare to gravity |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

ExtensionS

If Enterprise is operating at an angle other than horizontal or vertical, how does this affect the results of your analysis? Test it out. (Consider taking your apparatus through a full motion from horizontal to vertical to illustrate differences like are experienced on the real amusement park ride.)

Various accelerometers are available to use in this lab. One of the units that communicates wirelessly to your computing device is recommended. Vernier offers WDSS units, GDX-ACC and GDX-FOR units. All can communicate using Bluetooth technology.

Clarence Bakken & Roger Larson
April 2018