Resistors, Lights & LED’s – LabQuest+Comp

Introduction

The Vernier Power Amplifier (PAMP) is designed to boost signals from a LabQuest (LQ) or a LabPro for use in electronic and mechanical systems. It can amplify DC as well as several varieties of AC outputs and produce up to ±1 amp. This version of the lab will give specific instructions for using PAMP with a LabQuest and computer with Logger Pro, and give a variety of investigations that can be done. This lab is designed to take advantage of Power Amplifier features in Logger Pro 3.8 and LabQuest app 1.3 or later.

On/Off

Output

Current Monitor/ LabPro In

BTA-BTA

Power in

To PAMP

Mini-Phone to LabQuest

Vernier Power Amplifier

The mini-phone cord, is used to send signals from LabQuest to PAMP. It connects into the headphone output on the side of LabQuest. The BTA-BTA cord connects between PAMP and one of the analog inputs on the LabQuest, allowing the LabQuest to monitor the current being produced by PAMP.

Signals processed by the Power Amplifier come out of the binding posts on the front. The black post is an electrical ground point and connects to ground in the LabQuest and ground in a ±10 V voltage probe.

Purpose

Study the behavior of different electrical devices in dynamic circuits.

Equipment

Vernier Power Amplifier, LabQuest, Computer with Logger Pro, Voltage Probe (VP-BTA or DVP-BTA*), Resistors, Diode, LED’s. Light Bulb, Clip Leads

Procedure 1

- Connect the cords between Power Amplifier, LabQuest and computer as shown in the diagram that follows the lab. Connect your voltage probe to CH1 of the LabQuest. The lead colors in this and succeeding diagrams shows how to avoid ground problems when using VP-BTA.

- Connect the clip leads between the PAMP and the device as shown in this diagram, using a 330-ohm resistor as the device in the circuit:

From PAMP Output Posts

To VP-BTA or DVP-BTA

Device such as resistor or light bulb

- Turn the Power Amplifier on using the switch at the front. Turn LabQuest on. Launch Logger Pro. The screen on the LabQuest will show two arrows indicating the unit is connected to the computer. No readings will be taken on the LabQuest from here on.

- Above the data table and in the lower left-hand corner, you should see two quantities being measured, potential (voltage) and current. Click on Experiment > Set up Sensors > LabQuest.

- Click the [Power Amplifier] button, opening a new window to control the signal going into the Power Amplifier. Set the waveform to sine, frequency to 2 Hz, and Amplitude to 3 volts. Click the [Start] button. Slide this window down near the bottom of the screen. It will continue to be on, but you can access all the Logger Pro features.

- Click on the “stopwatch” icon to open the Data Collection setup screen. Set the rate to 50 samples/s and the length to 2 seconds. Click [Done] to put it away.

- Initiate data collection. After the one second of data collection, you will see two graphs, voltage versus time and current versus time. How do the shapes of these two graphs compare? What does that indicate about the relationship between current and voltage?

- On the potential versus time graph, click on the x-axis label, Time. In the menu that appears, choose Current. You will now have a graph of Potential versus Current. What is the shape of this graph? Does it confirm your thinking in step 6 above?

- Do a curve fit for this graph. What is the slope of the graph? Is this value related in any way to items in the experiment? To compare the next step with this one, click on Experiment > Store Latest Run.

- Change resistor value and repeat steps 7-9. Does this confirm your previous work? Make a general statement about the meaning of the slope on a Potential versus Current graph. Make a general statement about the shape of a Potential versus Current graph for a resistor.

# Procedure 2

- Replace the resistor that you used in the previous steps with a flashlight bulb.

- Open the window that sets the signal, the one you moved off screen earlier, and change the frequency to 0.2 Hz, voltage to 3 V. Then slide the window out of the way again.

- Click on the stopwatch icon and change the rate to 20 samples/s and the length to 5 seconds.

- Collect a fresh set of data using the light bulb. If you need to, change the graph to Potential versus Current. How does this graph compare with the one you obtained with a resistor? Does it tell you anything about the behavior of a hot light filament ?

- Repeat steps 12-14, but increase the frequency to 2 Hz, the sampling rate to 50 samples/s, and the length of data collection to 1 second. How does increasing the frequency of the signal change the shape of the graph? What is causing the change? How does the increased frequency affect the intensity of the light?

- Repeat steps 12-14, but increase the frequency to 10 Hz, the sampling rate to 100 samples/s, and the data collection length to 0.2 second. How does this change the shape of the graph? What is causing the change? How does the increased frequency affect the intensity of the light?

NOTE: Incandescent light bulbs are non-ohmic. From your experimentation, develop a meaning for this term and contrast that with what we mean when we say a device is ohmic.

# Procedure 3

- Click on Data > Clear All Data to get a fresh data table to work with in the next steps.

- Replace the light bulb that you used in the previous steps with a diode. The diode should have a line around the body that’s nearer to one end than the other. When placing it in your circuit, put the end with the line closer to the lead coming from the black output terminal.

Black terminal

Diode

Resistor

Red terminal

From PAMP Output Posts

To VP-BTA or DVP-BTA

Device such as resistor or diode or LED

Series Resistor

Connect a 330-ohm resistor in series with the diode for use in this lab. Connect the leads from your voltage probe across the diode as shown here:

- On the signal setup screen, set the waveform to sine, frequency to 2 Hz, voltage to 5 V.

- Click on the stopwatch icon, then change the rate to 50 samples/s and the length to 1 second.

- Collect a fresh set of data with the diode. Change the graphs to Potential versus Current if needed. Describe the graph you generated. Is the resistance of the diode constant like a resistor? Or does it resemble the light bulb? Or is it completely different? Describe the operation of the diode as well as you can.

- Diodes are used to convert alternating current (think sine wave) into pulsing direct current, current that flows in only one direction. How does your graph show this?

- Store the diode graph for future reference.

Shorter lead towards black terminal

# Procedure 4

- Replace the diode that you used in the previous steps with an LED, light emitting diode. LED’s typically have one lead that is shorter than the other. Place it closer to the black lead from the Power Amplifier like you placed the end of the diode with the line. When you connect it up, the LED should blink twice a second.

- Collect a fresh set of data with the LED. Change the graph to Potential versus Current if needed. Describe the graph you generated. Does the graph resemble the graph from the diode? How are they the same and how are they different?

- Store this graph, also, then change to a different color LED. How does the graph you obtain differ from the previous one? If you have a third LED, store the second and run a set of data for the third. Now compare all four diodes, but more particularly, the three LED’s. What changed as you moved from a red to yellow to green to blue LED? What changes as you move from red to yellow to green to blue light?

Analysis

- Answer the questions in the lab procedure.

- Determine the approximate resistance of your diode and LED’s when they were conducting current.

# Extension:

This lab calls for using the sine wave function to drive the circuits. Do you get the same results with a ramp function or square wave function? Try it and try to determine why you either get the same or why you get different results with different waveforms.

Connection Diagram

Circuit

BTA-BTA cable

VP-BTA

Clip Leads

CH1

CH2

PAMP

mini stereo

phone cable

USB-mini cable

to computer

*

Note: The two voltage probes sold by Vernier Software differ in one important characteristic. For VP-BTA, the ±10-volt voltage probe, the black lead is connected to electrical ground in the LabQuest. In this lab, it is also connected to the black output post on the Power Amplifier. On the other hand, for a DVP-BTA, the Differential Voltage Probe, the black lead is not connected to ground so the probe “floats” and measurements can be taken without worrying about ground connections.

Logger Pro must be version 3.8 or higher in order to interact with the Power Amplifier in the manner expected in this lab.

Clarence Bakken

Uploaded December 2009